/*!

* \file 	35_How_To.doc

* \brief 	SCCP New Feature Documentation

*

* Contains Documentation about Present Features.

*

* \author 	Diederik de Groot <dkgroot [at] talon.nl>

* \since	2009-11-12

* \note 	This program is free software and may be modified and distributed under the terms of the GNU Public License.

* \note 	You could read the documentation from this file; but it would probably

* 		be easier to go to http://chan-sccp-b.sourceforge.net

*/

/*!

* \page _howto How To

*

*  - \ref nf_shared_lines 	"How to use Shared Lines on a device"

*  - \ref nf_phone_buttons 	"How to set Phone Buttons on a device (softkeys)"

*  - \ref nf_hotline		"How to use the Hotline option on a device"

*  - \ref nf_adhoc_plar		"How to use the adhoc option on a line (PLAR)"

*  - \ref nf_dynamic_speeddial   	"How to use Dynamic SpeedDials"

*  - \ref nf_sccp_softkeys   	"How to use the Soft Keys on the Phone"

*  - \ref nf_sccp_reload   	"How to use Reload"

*  - \ref nf_sccp_function   	"How to use Dialplan Function"

*  - \ref nf_devicestate		"How to use device state"

*  - \ref nf_digittimeout		"How does digit timeout and dialing work"

*  .

* <HR>

*

*

* \section nf_shared_lines Shared Lines

*

* Shared lines is simple, and is activated readily by registering multiple phones on the same line.

* That's all, no extra options required. What happens is that all phones on a shared line "see" the calls of

* the other phones and vice versa. If you have a call on a shared line, just press "hold" on one phone

* and resume that call with "resume" on another phone.

*

* Earlier, without shared lines, registering multiple phones on the same line lead to an error

* that the line was already in use. Now, you simply get a shared line.

* <HR>

*

*

*

*

*

*

*

* \section nf_phone_buttons Phone Buttons

*

* We do not use the different autologin, speeddial options any longer, instead trunk uses a unified scheme for adding buttons:

* Each button is added to the phone in the order as specified in the config file by specifying a "button" assignment. The syntax is

* button = <empty|line|speeddial|feature|service>[, name[, option1[, option2]]]

*

* buttons come in the following flavours:

*   - empty: Empty button (no options)

*   - line: Registers the line with identifier specified as [name]

*   - speeddial: Adds a speeddial with label [name] and [option1] as number

Optionally, [option2] can be used to specify a hint by extension\@context as usual.

*   - service: Adds a service url

*   - Feature buttons have an on/off status represented on the device with a tick-box and can be used to set the device in a particular state.

*     Currently Possible [option1],[option2] combinations:

*      - privacy,callpresent		= Make a private call, number is suppressed

*      - privacy,hint			= Make a private call, hint is suppressed

*      - cfwdall,number		= Forward all calls

*      - cfwbusy,number		= Forward on busy

*      - cfwnoaswer,number		= Forward on no-answer (not implemented yet)

*      - DND,busy			= Do-not-disturb, return Busy signal to Caller

*      - DND,silent			= Do-not-disturb, return nothing to caller

*      - monitor			= Record Calls using AutoMon (asterisk 1.6.x only)

*      - devstate,custom_devstate	= Device State Feature Button (asterisk 1.6.1 and up).

custom_devstate is the name of the custom devicestate to be toggled (\ref nf_devicestate)

*      - hold				= To be implemented

*      - transfer			= To be implemented

*      - multiblink			= To be implemented

*      - mobility			= To be implemented

*      - conference			= To be implemented

*      .

*   .

*

* In a configuration file this could look like:

* \code

* button = line, 519

* button = line, 520@01: Phone1

* button = line, 521@02: Phone1!silent

* button = line, 522@03: Phone1, default

* button = empty

* button = speeddial, "Cantine", 332, 332@hints

* button = service, "Asterisk", "www.asterisk.org"

* button = feature, "privacy callpresent", privacy, callpresent

* button = feature, "callforward busy", cfwdbusy, 529

* button = feature, "DND busy", "DND", "busy"

* button = feature, "Record Calls", monitor

* \endcode

*

* <HR>

*

* For more information about hints check \ref nf_hints here

*

*

*

*

*

*

* \section nf_dynamic_speeddial How to use Dynamic SpeedDials (now on by default)

*

* Dynamic SpeedDials give additional information about the status of the lines assigned the speeddial. The

* Icon displayed changes to indicate "IDLE", "DND", "IN USE" and "RINGING". In the latest version this feature is

*

* To use Normal SpeedDials(olod behaviour) instead of Dynamic SpeedDials you need to use a configure parameters during compilation

* \code

* ./configure --disable-speeddial

* \endcode

*

* --disable-dynamic-speeddial = Do not use any dynamic speeddial

* --disable-dynamic-speeddial-cid = Do not display the calledid information on the dynamic speeddial

*

* Dynamic SpeedDial is now switched on by default !!.

* <HR>

*

*

*

*

*

* \section nf_hotline How to use the hotline option

*

* Setting the hotline Feature on a device, will make it connect to a predefined extension as soon as the Receiver

* is picked up or the "New Call" Button is pressed. No number has to be given. This works even on devices which

* have no entry in the config file or realtime database.

*

* The hotline function can be used in different circumstances, for example at a door, where you want people to be

* able to only call one number, or for unprovisioned phones to only be able to call the helpdesk to get their phone

* set up. If hotline_enabled = yes, any device which is not included in the configuration explicitly will be allowed

* to registered as a guest device. All such devices will register on a single shared line called "hotline".

*

* For example:

* \code

*   - hotline_enabled=yes

*   - hotline_context=default

*   - hotline_extension=111

* \endcode

* This will add a Hotline on all phones, dialing 111 via context default.

* <HR>

*

*

*

*

*

*

* \section nf_adhoc_plar How to use the adhoc option (PLAR)

*

* Adhoc Number or Private-line automatic ringdown (PLAR):

* Adhoc/PLAR circuits have statically configured endpoints and do not require the user dialing to connect calls.

*

* - The adhocNumber is dialed as soon as the Phone is taken off-hook or when the new-call button is pressed

* - The adhocNumber will <b>not</b> be dialed when choosing a line; so when you choose a line you can enter a number manually.

*

* For example adding this to the line configuration:

* \code

*   adhocNumber = 818

* \endcode

* Will automatically dial 818 as soon as the Receiver is picked up or the new call button is pressed.

* <HR>

*

*

*

*

*

*

* \section nf_sccp_softkeys 	Soft Keys Howto

*

* \section sk_config		Softkey Configuration

*

* You can create several softkeyset in you config for use on different devices. You can assign these softkeyset to a device

* using: softkeyset = name_of_softkeyset in the device section of the config.

* (<a href="http://tools.cisco.com/search/display?url=http%3A%2F%2Fwww.cisco.com%2Fen%2FUS%2Fdocs%2Fvoice_ip_comm%2Fcuipph%2F7975G_7971g-ge_7970g_7965g_7945g%2F8_5%2Fenglish%2Fquick_reference%2F7975qrcrd85.pdf&pos=1&strqueryid=1&websessionid=Baq5yYY1J3zwB6fskFWXc2e">Softkey explanations / logo's</a>)

*

* for example:

* \code

* [SEP001122334455]

* devicetype=7960

* ...

* softkeyset = mysoftkeyset

*

* [mysoftkeyset]

* type=softkeyset

* onhook      = redial,newcall,cfwdall,dnd

* connected   = hold,endcall,park,select,cfwdall,cfwdbusy,idivert

* onhold      = resume,newcall,endcall,transfer,confrn,select,dirtrfr,idivert

* ringin      = answer,endcall,idivert

* offhook     = redial,endcall,private,cfwdall,cfwdbusy,pickup,gpickup,meetme,barge

* conntrans   = hold,endcall,transfer,confrn,park,select,dirtrfr,cfwdall,cfwdbusy

* digitsfoll  = back,endcall

* connconf    = hold,endcall,join

* ringout     = endcall,transfer,cfwdall,idivert

* offhookfeat = redial,endcall

* onhint      = pickup,barge

* \endcode

*

* The [mysoftkeyset] section about contains the state of the phone as the first part (before the equal sign) and the

* softbuttons shown during this state in the second part.

*

* Multiple softkey set can be created and attached to different devices.

*

* \subsection sk_softkeys_avail	These are the softkeys available

* \subsubsection sk_redial 		How to use the Redial Softkey

*

* Using the Redial Button you can Redial the last Called Number on this Line or Device

*

* The last number called is stored in Asterisks local database (AstDB) before a module unload, so the state can be restored to previous setting on a module reload or restart of Asterisk.

*

* \subsubsection sk_newcall 		How to use the NewCall Softkey

*

* Using the New Call Button you can initiate a new Call just like picking up the receiver.

* You can either type in the number to be called before or afterwards.

*

*

* \subsubsection sk_hold 		How to use the Hold Softkey

*

* Using the Hold Button you can hold the current line and undertake some other action

* You can later resume the held line use \ref sk_resume

*

*

* \subsubsection sk_resume 		How to use the Resume Softkey

*

* Using the Resume Button you can resume a previously help line which has been put on hold using \ref sk_hold

*

*

* \subsubsection sk_transfer	 	How to use the Transfer Softkey

*

* Using the Transfer 			Button you can Transfer the Currentline to another Number

* 					You can also use a speeddial during transfer

*

* \subsubsection sk_endcall 		How to use the End Call Softkey

*

* Using the End Call Button you can End the Call on the Current Line

*

*

* \subsubsection sk_dnd 		How to use the Do Not Disturb (DND) Softkey

*

* Using the Do Not Disturb (DND) Button you set your device/line to the Do Not Disturb Status.

* When receiving a call your phone will not ring and the People Call You Will get a Busy Signal

*

* The status of DND is stored in Asterisks local database (AstDB), so the state of DND can be restored to previous setting on a module reload or restart of Asterisk.

*

* \subsubsection sk_backspace 	How to use the BackSpace (<<) Softkey

*

* Using the backspace (<<) Button you can erase the last entered digit/character

*

*

* \subsubsection sk_answer	 	How to use the Answer Softkey

*

* Using the Answer Button you can Answer an Incoming Call

*

*

* \subsubsection sk_select	 	How to use the select softkey

*

* The "Select" softkey is used for bridging two channels (redirect).

* Select your first channel and press the select softkey. On the display this channel is marked

* with a checkmark.

* By selecting the second channel, it is also marked with a checkmark and DirTrFr (\ref sk_dirtrfr)

* is enabled. Press this key to bridge both channels.

*

*

* \subsubsection sk_dirtrfr	 	How to use the DirTrfr

*

* When two channels have been selected using the Select SoftKey (\ref sk_select) then

* using the DirTrfr SoftKey can be used to Connect Both Lines to one another (bridging both channels).

*

*

* \subsubsection sk_cfwdall	 	How to use the Call Forward All Button

*

* Using this Button you can set the current line / device to forward all incoming call to another number

* This number can be another SCCP/SIP/IAX Device or even an external number (if permitted to call externally)

*

* The status of CallForward is stored in Asterisks local database (AstDB), so the state can be restored to previous setting on a module reload or restart of Asterisk.

*

* \subsubsection sk_cfwdbusy	 	How to use the Call Forward if Busy Button

*

* Using this Button you can set the current line / device to forward incoming calls when busy to another number

* This number can be another SCCP/SIP/IAX Device or even an external number (if permitted to call externally)

*

* The status of CallForward is stored in Asterisks local database (AstDB), so the state can be restored to previous setting on a module reload or restart of Asterisk.

*

* \subsubsection sk_cfwdnoanswer 	How to use the Call Forward if No-Answer Button

*

* Using this Button you can set the current line / device to forward incoming calls when you don't/can't answer to another number

* This number can be another SCCP/SIP/IAX Device or even an external number (if permitted to call externally)

*

* The status of CallForward is stored in Asterisks local database (AstDB), so the state can be restored to previous setting on a module reload or restart of Asterisk.

*

* \subsubsection sk_park	 	How to use the Park Button

*

* Using the Park Button make it possible to Send a Call to the Asterisk Parking Lot

* Channels parked in the Parking Lot can be Picked-Up by Another Phone Using the Pickup Button or the Pickup Number

* Configured in features.conf under pickupexten

*

*

* \subsubsection sk_transfer_button	 	How to use the Transfer Button

*

* Using the Transfer Button gives you the possibily to Transfer the Current Line to a New Number

* After pressing the Transfer button you are asked to Enter a New Number to which the Line will have to be transfered

*

*

* \subsubsection sk_private	 	How to use the Private Button

*

* Using the Private Button before dialing a New Outside Line will Prevent the Number dialed to be monitored by

* Devices that use a HINT. It does not prevent the number turning up in Asterisk Logging or the Console Window.

*

*

* \subsubsection sk_conference 	How to use the Conference Button

*

* Using the Conference Button makes it possible to set up a Simple Three Way Conference. There are two ways this can be done:

* Option 1: You Already have to Lines Occupied (One is active and the other Held), which you would like to put in a Conference; Simple Press the Conference Button

* Option 2: You have one line Occupied and would like to call Someone else and Join them all into a Conference;

*           Press the Conference Button and you will be asked for the 3 party number which need to be added to the conference,

*           once this line is connected, you press the Conference Button Again and the Conference Starts.

* \subsubsection sk_conflist	 	How to use the Conference List Button

*

* Using the Conference List Button shows you a list of members of the current conference you are in with the mute status. If you are the moderator

* of this conference you have the ability to mute/unmute and kick members out of the conference.

*

* \subsubsection sk_join	 	How to use the Join Button

*

* Using the Join Button makes it possible to Add another member/line to an already running Conference

* When you are in a conference and you receive an Incoming Call you can join this channel/line to the running Conference

*

*

* \subsubsection sk_barge	 	How to use the Barge Button

*

* Using the Barge Button makes it possible to listen in on an On-Going Call on another Line / Device

*

*

* \subsubsection sk_cbarge	 	How to use the Conference Barge Button

*

* Using the Barge Button makes it possible to listen in on an On-Going Conference on another Channel

*

*

* \subsubsection sk_meetme	 	How to use the Meetme Button

*

* Using the Conference Button makes it possible to set up a Simple Three Way Conference. There are two ways this can be done:

* Option 1: You Already have to Lines Occupied (One is active and the other Held), which you would like to put in a Conference; Simple Press the Conference Button

* Option 2: You have one line Occupied and would like to call Someone else and Join them all into a Conference;

*           Press the Conference Button and you will be asked for the 3 party number which need to be added to the conference,

*           once this line is connected, you press the Conference Button Again and the Conference Starts.

* \sa sk_conference

*

*

* \subsubsection sk_pickup	 	How to use the Pickup Button

*

* Using the Pickup Button Makes it possible to Pickup a Parked Call from the Parking Lot

* This is equivalent to using the keys defined in features.conf under pickupexten

*

*

* \subsubsection sk_gpickup	 	How to use the Group Pickup Button

*

* Using the GroupPickup Button Makes it possible to Pickup a Ringing Line in defined in your PickupGroup

* This is equivalent to using the keys defined in features.conf under pickupexten

* <HR>

*

*

*

*

*

*

*

* \section nf_sccp_reload	 	Reload Howto

*

* When chan-sccp-b has been configured with the --with-dynamic-config during the build fase. The new "sccp reload" function

* will be availble in from the CLI inside asterisk. When run, chan_sccp-b will reload the sccp.conf file and update all it's

* structures. When necessary phones will be sent a reset to make them reload their configuration. When a phone is currently

* in used, but would need to be reset, it will reset as soon as the current call is hungup.

*

*

*

*

*

*

* \section nf_sccp_function	 	How to use dialplan functions

*

* We have added new dialplan functions

* \ref nf_sccp_dialplan_sccpdevice SCCPDEVICE, \ref nf_sccp_dialplan_sccpline SCCPLINE and nf_sccp_dialplan_sccpchannel SCCPCHANNEL

* to provide the ability to ask information about sccp device, sccp lines and sccp channels.

*

* And \ref nf_sccp_dialplan_sccpsetcodec SCCPSetCodec, \ref nf_sccp_dialplan_sccpsetcalledparty SCCPSetCalledParty and \ref nf_sccp_dialplan_sccpsetmessage SCCPSetMessage

* to provide the ability to change chan_sccp behaviour while prosessing dialplan requests.

*

* \subsection nf_sccp_dialplan_sccpdevice SCCPDEVICE

*

* Usage: SCCPDEVICE([deviceId|current],[option]);

* Use: core show functions SCCPDEVICE to find out all the options

*

* the first param can be either "current" or a device ID like "SEP????????????"

*

* the second param defines the information to be retrieved and can be on of these:

* ip, id, status, description, config_type, skinny_type, tz_offset, image_version, accessory_status, registration_state

* codecs, state, lines_registered, lines_count, last_number, capability, early_rtp, channel_count, supported_protocol_version

* used_protocol_version, mwi_light, dynamic|realtime, active_channel, transfer_channel,

* conference_id, allow_conference, conf_play_general_announce, allow_conference,

* conf_play_part_announce, conf_mute_on_entry, conf_music_on_hold_class,

* current_line button_config, pending_update, pending_delete, chanvar[varname], codec[codecname]

*

* Within the dialplan you can use the function SCCPDEVICE to retrieve information about an SCCP Device like this:

* \code

* Set(my_ip=${SCCPDEVICE(current,ip)});

* Set(my_id=${SCCPDEVICE(current,id)});

* Set(my_codecs=${SCCPDEVICE(current,codecs)});

* Set(my_codec_g792=${SCCPDEVICE(current,codec[g792])});

* Set(my_button_config=${SCCPDEVICE(current,button_config)});

* Set(other_button_config=${SCCPDEVICE(SEP001122334455,button_config)});

* \endcode

* to put the information of a/the device in a dialplan variable.

*

* \subsection nf_sccp_dialplan_sccpline SCCPLINE

*

* Usage: SCCPLINE([current|parent|linename],[option]);

* Use: core show functions SCCPLINE to find out all the options

*

* the first param can be either "current", "parent" or a line name assigned to the line.

* "parent" refers to the line who forwarded the call to you and is only filled if this channel has been forwarded.

*

* the second param defines the information to be retrieved and can be on of these:

* id, name, description, label, vmnum, trnsfvm, meetme, meetmenum, meetmeopts, context, language, accountcode,

* musicclass, amaflags, callgroup, pickupgroup, cid_name, cid_num, incoming_limit, channel_count, dynamic|realtime,

* pending_delete, pending_update, regexten, regcontext, adhoc_number, newmsgs, oldmsgs, num_lines, lines, chanvar[varname]

*

* Within the dialplan you can use the function SCCPLINE to retrieve information about an SCCP Line like this:

* \code

* Set(my_line_name=${SCCPDEVICE(current,name)});

* Set(my_line_descr=${SCCPLINE(current,description)});

* Set(my_line_devices=${SCCPLINE(current,devices)});

* Set(parent_line_id=${SCCPLINE(parent,id)});

* Set(other_line_id=${SCCPLINE(98011,id)});

* Set(other_line_devices=${SCCPLINE(98011,devices)});

* \endcode

* to put the information of a/the line in a dialplan variable.

*

* \subsection nf_sccp_dialplan_sccpchannel SCCPCHANNEL

*

* Usage: SCCPCHANNEL([current|callid],[option]);

* Use: core show functions SCCPCHANNEL to find out all the options

*

* the first param can be either "current" or a callid assigned to the channel.

*

* the second param defines the information to be retrieved and can be on of these:

* callid, format, isCodecFix, codecs, capability, calledPartyName, calledPartyNumber, callingPartyName, callingPartyNumber

* originalCallingPartyName, originalCallingPartyNumber, originalCalledPartyName, originalCalledPartyNumber, lastRedirectingPartyName

* lastRedirectingPartyNumber, cgpnVoiceMailbox, cdpnVoiceMailbox, originalCdpnVoiceMailbox, lastRedirectingVoiceMailbox,

* passthrupartyid, state, previous_state, calltype, dialed_number, device, line, answered_elsewhere, privacy, monitorEnabled, parent,

* recvip, peerip

*

* De carefull with SCCPCHANNEL, When calling this you must provide a valid SCCP Channel. So check if the channel is of

* type SCCP before calling this function !

*

* Within the dialplan you can use the function SCCPCHANNEL to retrieve information about an SCCP channel like this:

* \code

* Set(my_channel_callid=${SCCPCHANNEL(current,callid)});

* Set(my_channel_device=${SCCPCHANNEL(current,device)});

* Set(my_channel_line=${SCCPCHANNEL(current,line)});

* Set(my_channel_capability=${SCCPCHANNEL(current,capability)});

* Set(my_channel_called=${SCCPCHANNEL(current,calledPartyName)});

* Set(my_channel_calling=${SCCPCHANNEL(current,callingPartyName)});

* Set(my_channel_state=${SCCPCHANNEL(current,state)});

* Set(my_channel_calltype=${SCCPCHANNEL(current,calltype)});

* Set(my_channel_dialed_number=${SCCPCHANNEL(current,dialed_number)});

* Set(other_channel_dialed_number=${SCCPCHANNEL(4,dialed_number)});

* \endcode

* to put the information of the/a channel in a dialplan variable.

*

* \subsection nf_sccp_dialplan_sccpchannel1 Generic CHANNEL

*

* Generic Dialplan Cmd CHANNEL information provided when used with an SCCP channel.

*

* Usage: CHANNEL([option]);

*

* Possible Options:

*  - recvip: Current IP-Addres of the Connection to the Phone

*  - peerip: Real IP-Address of the Phone in question (Could be different if the phone is on a Natted Connection)

*  -  from: device type

*  - useragent: device name (a.k.a SEP00123456789)

*  .

*

*

* Dialplan function to manipulate sccp channels and dial commands are:

* \subsection nf_sccp_dialplan_sccpsetcodec SCCPSetCodec

* Usage: SCCPSetCodec(codec)

* Sets the preferred codec for dialing out with the next DIAL statemnet using an sccp channel.

*

* SCCPSetCodec(codec) is now deprecated in favour of generic channel method 'Set(CHANNEL(codec)=alaw)'. Please do not use SCCPSetCodec anymore.

*

* \subsection nf_sccp_dialplan_sccpsetcalledparty SCCPSetCalledParty

* Usage: SCCPSetCalledParty("name" \<ext\>)

* Sets the name and number of the called party before using the DIAL statement, only usefull when dialing an SCCP Channel.

*

* Previous function name SetCalledParty(codec) is now deprecated in favour of SCCPSetCalledParty.

*

* \subsection nf_sccp_dialplan_sccpsetmessage SCCPSetMessage

* Usage: SCCPSetMessage(<message text> [,timeout])

* Send a Message to the Calling Device (and remove after timeout, if timeout is ommited will stay until next/empty message.

*

*

* Previous function name SetMessage(codec) is now deprecated in favour of SCCPSetMessage.

*

*

*

*

*

*

* \section nf_devicestate         How to use devicestate

*

* When chan-sccp-b is used with asterisk 1.6.1 and up, you have the option to use configure directive --enable-devstate-feature.

* This will make chan-sccp-b report back the device_state back to asterisk, to be used by for example the

* Dialplan DEVICE_STATE function as such:

*

* exten => s,1,NoOp(My phone state is currently ${DEVICE_STATE(SCCP/98011)})

*

* \subsection nf_customdevicestate	Custom Device State

*

* If you add a new devstate button to you sccp.conf like so, you will have a new custom device state which can be set and monitored

* \code

* button = feature, Coffee is Ready, devstate, coffee

* \endcode

*

* When you press this button the device state will go from NOT_INUSE to INUSE and back again

*

* You can then switch the feature devstate button on and of from the dialplan with

* \code

* exten => s,1,Set(DEVICE_STATE(Custom:coffee)=INUSE)

* \endcode

* To signal everyone that the coffee is ready.

*

* From the console(CLI) this can be handled via:

* \code

* devstate change Custom:coffee INUSE

* \endcode

*

* The new phones models with lighted buttons will nicely light up their buttons upon "INUSE".

*

* You can read more about customer device states <a href="http://www.asterisk.org/node/48325">here</a>.

*

*

*

*

* \section nf_digittimeout	      Digit timeout and dialing

*

* Dialing changes:

*

* - When entering the number with the receiver down, and then pressing dial or picking up, we calculate the standard

*   deviatoin between the timing of the entered numbers. If the all fall withing the same pattern (about 100 milliseconds

*   per digit) with a std-dev les then 3.5 we can assume the numbers are not entered by hand and emulate enbloc dialing

*   and reduce the total digittimeout to 2000 milliseconds (2 secs after the last digit). The following two rules still apply.

* - Matching of number uses regcontext group specified in sccp.conf first, if a full match is found, the digittimout is halved.

* - If a full match is found in the dialplan, and no other dialplan entries would match, dialing starts immediatly.

*

*

*/